# EECE573: Assignment #5

## Tejas Randeria

# CAUTION: Please use Xilinx to run the files, it’ll not work in Modelsim
All the files included have to present in the same folder

# Objective:-

The assignment divided into 4 parts

- Part A: FIFO

  - To create a FIFO module (size should be parameterized and tested for more than 1 size). Make sure to find corner cases (e.g. full, empty, rollover from bottom to top). Position pointers must be self-contained in the module. Make sure to properly initialize them.

- Part B: LIFO/Stack

  - To create a LIFO module (size should be parameterized and tested for more than 1 size). Make sure to find corner cases (e.g. full, empty, rollover from bottom to top). Position pointers must be self-contained in the module. Make sure to properly initialize them.

- Part C: Synthesize each of the above queues of 4 4-bit entries. Show analysis for speed/area/power under the following constraints:

  - Minimum area

  - Minimum latency

  - Minimum power

- Part D: Synthesis

  - Create a synthesized version of the 16-bit ripple-carry adder from Assignment #2, using Design Vision

  - Repeat the process for the module “simple_adder.v” from Blackboard.

  - Compare the final area/speed numbers for the RCA and the simple_adder.

# Description:-

- Part A: FIFO

  - All the variables that are present in the code have been named to be self-explanatory such as

    - vector_in: the input vector to the fifo_top module

    - reset: to get the system to its initial state

    - clk: synchronous system

    - data_out: the requested data

    - empty_flag, full_flag: denoting the current status of the fifo

  - The vector_in has been segregated and stored in two regs

    - control_in: the first 2 bits deciding the type of operation to be performed

    - data_in: used only during write operation, else not used

- Part B: LIFO/Stack

  - All the variables that are present in the code have been named to be self-explanatory such as

    - vector_in: the input vector to the lifo_top module

    - reset: to get the system to its initial state

    - clk: synchronous system

    - data_out: the requested data

    - empty_flag, full_flag: denoting the current status of the lifo

  - The vector_in has been segregated and stored in two regs

    - control_in: the first 2 bits deciding the type of operation to be performed

  - data_in: used only during write operation, else not used

- Part C: Synthesized the queues and reported the results below

- Part D: Synthesized the adder modules and reported the results below

# Working:-

- Part A: FIFO

  - Being a synchronous design, we are using clock. The reset signal is used to initialize the module.

  - The status (empty and full flags) is being processed first, effecting the next process taking place i.e. whether to write or read from the fifo

  - fifo_valid_invalid_bit – denotes the validity of a particular location

  - fifo_data – contains the data present in the fifo

  - If the fifo is empty, data cannot be read and if the fifo is full, data cannot be written; the status flags notify if the above operations can take place

  - Additional conditions have to be met

    - FIFO full – denoted by the head and tail pointer pointing at the same location containing valid data

    - FIFO empty – denoted by the head and tail pointer pointing at the same location containing invalid data

  - In this case roll over can occur as the read and write pointer are moving in the same direction for a read and write

  - As we read, invalidation of the location occurs hence data can be written on it if overflow occurs

  - Write pointer always points to the next available location to be written and Read pointer always points to the last written valid location

- Part B: LIFO

  - Being a synchronous design, we are using clock. The reset signal is used to initialize the module.

  - The status (empty and full flags) is being processed first, effecting the next process taking place i.e. whether to write or read from the lifo

  - lifo_valid_invalid_bit – denotes the validity of a particular location

  - lifo_data – contains the data present in the lifo

  - If the lifo is empty, data cannot be read and if the lifo is full, data cannot be written; the status flags notify if the above operations can take place

  - Additional conditions have to be met

    - LIFO full – denoted by the head and tail pointer pointing at the same location containing valid data

    - LIFO empty – denoted by the head and tail pointer pointing at the same location containing invalid data

  - In this case roll over cannot occur as the read and write operations have a different functionality

# Conclusion:-

- Part A: FIFO

Figure 1 FIFO Reset & Write operation

My test bench is extensive and covers all the cases as well as corner cases

Figure 2 FIFO Empty & Full Flags

Figure 3 FIFO Read & More Write Operations

Figure 4 LIFO Read & More Write Operations

Figure 5 LIFO Reset & Write Operations

Part B: LIFO

Figure 6 LIFO Empty & Full Flags

- Part C: Synthesis Analysis (Queues)

  - After each analysis removed the design and loaded it again

  - FIFO

    - Considering no optimization is applied while compiling the design

| Analysis (No optimization) | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| FIFO | 0.86 ns | 1559 | 18.09 uW | 7 uW |

    - Optimizing for Timing

      - The minimum I could get with the optimizations was 0.86ns

    - Optimizing for Area

      - Settings the max area - 0

      - Compiling with high map effort and area effort

      - Decreases the area by a small amount while increasing the dynamic power

| Analysis (Area Optimized) | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| FIFO | 0.86 ns | 1552.7 | 19.12 uW | 7.03 uW |

    - Optimizing for Power

      - Setting the dynamic to 10uW

      - Compiling with high power effort, low area effort and high map effort

      - Increases the longest path timing by a small amount and decreases the dynamic power by a small amount

| Analysis (Power Optimized) | Tming (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| FIFO | 0.90 ns | 1572.13 | 17.94 uW | 7 uW |

  - LIFO

    - Considering no optimization is applied while compiling the design

| Analysis (No Optimization) | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| LIFO | 0.88 ns | 1618.15 | 19.87 uW | 7 uW |

    - Optimizing for Timing

      - The minimum I could get with the optimizations was 0.88ns

    - Optimizing for Area

      - Settings the max area - 0

      - Compiling with high map effort and area effort

| Analysis (Area Optimized) | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| LIFO | 0.91 ns | 1626 | 21.04 uW | 7.2 uW |

    - Optimizing for Power

      - Setting the dynamic to 10uW

      - Compiling with high power effort, low area effort and high map effort

      - Increases the longest path timing by a small amount and decreases the dynamic power by a small amount

| Analysis (Power Optimized) | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| LIFO | 0.90 ns | 1642.12 | 19.21 uW | 7 uW |

- Part D: Synthesis Analysis (RCA and Simple adder)

  - Ripple Carry Adder (RCA)

    - Optimizing for Area and Timing

      - Settings the max area - 0

      - Compiling with high map effort and area effort

| Analysis | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| RCA | 2.14 ns | 645.446 | 206.0775 uW | 3.1385 uW |

  - Simple Adder

    - Optimizing for Area and Timing

      - Settings the max area - 0

      - Compiling with high map effort and area effort

| Analysis | Timing (Arrival) | Total Area | Dynamic power | Leakage power |
| --- | --- | --- | --- | --- |
| Simple Adder | 4.10 ns | 472.84 | 133.42 uW | 2.27 uW |

  - The clock frequency for the RCA = (1/Tming) = (1/2.14ns) = 467 Mhz

  - The clock frequency for the Simple Adder = 243 Mhz

Figure 8 Simple Adder Schematic

Figure 7 RCA Schematic

  - As we can see from the schematic, the gate implementation is very different

  - Due to this varied implementation the longest timing path of the adders is different. Now considering this particular technology, the simple adder implementation introduces a lot of the delay in the circuit. Whereas the RCA circuit has a smaller longest path delay compared to the simple adder

  - Even though the area requirements of the simple adder is less it is severely affected by the longest path delay and dynamic power

# Feedback:-

- Tejas Randeria

  - Implemented the lifo and fifo queues in Verilog

  - More understanding of the synopsys tool

  - Understood the various tools in optimizing the circuit for various parameters